FIXED WING VTOL DRONE – FUNCTIONAL and TECHNICAL REQUIREMENTS

DESCRIPTION

The UAV shall be a purely electric heavy-lift VTOL drone designed for LiDAR/photo mapping, long-distance cargo delivery, transport, and security surveillance. The drone shall be equipped with four lift motors and two fixed-wing motors, enabling vertical take-off like a quadcopter. Upon reaching a specified altitude, the drone shall transition to horizontal flight as a push-prop fixed-wing aircraft.

The drone shall be capable of a flight endurance of up to 4 hours with a payload of 2 kg, and at least 1 hour with a payload of 8 kg. A separate bin shall be provided for gimbal camera installation, allowing target search via onboard camera when carrying a heavy payload.

The VTOL drone shall be weatherproof and capable of operating in adverse weather conditions, including rain, to perform urgent missions such as search and rescue. The fixed-wing UAV shall incorporate advanced quick-assembly technology on both airframe and cable connectors, enabling rapid assembly and disassembly.

The drone shall be equipped with onboard aviation lights for identification in flight, including during night operations. Ground control software (LAUNCH) shall be provided, designed for ease of use by beginners, enabling operation without prior flight experience. Landing shall be supported by a range detection radar device, ensuring gentle and safe descent.

The UAV shall not require compass calibration, saving time in emergency missions. The compass shall be replaced by DGPS, ensuring immunity to electromagnetic interference (EMI) and significantly improving flight stability.

An optional Follow Me feature shall be available: in missions where the flight platform (such as a boat or vehicle) is moving, the drone shall be capable of following the platform and landing upon mission completion. This function shall be particularly useful for surveillance and monitoring operations.

TECHNICAL PARAMETERS

| Parameter | Value |
| --- | --- |
| Wingspan when ready to fly | Between 4.0 and 5.0m |
| Payload | >= 8kg |
| Endurance | >= 1h with 8kg payload |
| Cruise speed | >= 80km/h |
| Wind tollerance in flight | >40 km/h |
| Operating altitude | >=3000m |

ADDITIONAL REQUIREMENTS

The UAV shall allow quick battery replacement to ensure rapid mission turnaround.

The UAV shall be supplied with a remote-control unit, a battery charger, and two (2) additional battery packs (three (3) in total).

Maintenance instructions shall be provided with the offer.