# SERVICE TRAILERS WITH MOBILE TELECOMMUNICATION MAST – FUNCTIONAL and TECHNICAL REQUIREMENTS

Three (3) service trailers shall be supplied, serving as mobile units for the installation of telecommunication equipment.

DESCRIPTION

Each trailer shall be equipped with the following key components:

Telescopic mast: A 10-meter telescopic mast enabling the placement of antennas at an appropriate height. The mast shall be capable of operating in the fully extended state without the need for side guy wires fixed to anchors outside the trailer. The lifting and lowering mechanism of the mast may be pneumatic, mechanical using steel cables, or screw-driven. The system shall ensure safe operation even when the mast is partially extended.

Diesel generator: A 5 kW diesel generator shall be provided for powering the equipment.

Cabinet for telecommunication equipment: The cabinet shall comply with IP65 standards, have an internal width of 19 inches with appropriate side mounting rails, and a depth in accordance with ETSI requirements. Forced ventilation during operation shall be provided.

Antenna rotators:
  - One rotator shall provide azimuth and elevation control for a directional parabolic antenna for uplink.
  - A second rotator shall provide azimuth control for a sector antenna for 5G frequency 2.3 GHz (N40).

Starlink connection: The trailer shall be equipped with an integrated Starlink connection to ensure backup connectivity.

UPS (Uninterruptible Power Supply): A UPS with at least 30 minutes of autonomy shall be provided to ensure uninterrupted operation during refueling of the generator.

Electrical wiring: The trailer shall allow connection to an external 220 VAC power supply. An automatic transfer switch shall toggle between external supply and the generator, with all power always routed through the UPS.

Control cabinet: A separate cabinet shall be provided for the equipment controlling the rotators and the raising/lowering of the mast.

TECHNICAL REQUIREMENTS OF THE TRAILER

- Total weight: The total weight shall not exceed 750 kg.

- Brake: The trailer shall be equipped with an overrun brake.

- Stability: The trailer shall be equipped with extendable legs to ensure stability during operation.

- Construction: The trailer shall be manufactured from galvanized structural elements.

- Hitch: The trailer shall be fitted with a 50 mm ISO towing hook (Class A – UNECE Regulation No. 55).

- Electrical connector: The trailer shall be equipped with a 13-pin standard electrical connector.

- Certification: The trailer shall be certified for road use.